# CAN Driver Software Requirements Specification
AUTOSAR Classic Platform Release R24-11

7. CAN-A05 - Interrupt Control and Write Polling	1

7.1 Scope Summary	2

7.2 Applicable SWS Requirements	2

7.3 Project SRS Requirements	3

7.3.1 CAN_SRS_049  - Disable Enabled Controller Interrupts	3

7.3.2 CAN_SRS_050  - Balance Nested Interrupt-Control Calls	3

7.3.3 CAN_SRS_051  - Track Controller Interrupt State Changes	4

7.3.4 CAN_SRS_052  - Restore Allowed Controller Interrupts	4

7.3.5 CAN_SRS_053  - Ignore Enable Without Prior Disable	5

7.3.6 CAN_SRS_054  - Provide Reentrant Interrupt Control	5

7.4 Supporting Artifact	6

7.5 Open Points and Assumptions	6

# 7. CAN-A05 - Interrupt Control and Write Polling

| Section owner(s) | SonNT208 |
| --- | --- |
| Technical scope | CAN controller interrupt control and configured transmit-confirmation polling, including Can_DisableControllerInterrupts, Can_EnableControllerInterrupts, nested interrupt-disable handling, restoration of allowed interrupts, Can_MainFunction_Write, configured polling behavior, and transmit confirmation notification. |
| Primary SWS/API references | Can_DisableControllerInterrupts; Can_EnableControllerInterrupts; Can_MainFunction_Write; CanIf_TxConfirmation; controller interrupt state; nested interrupt-disable counter; CanTxProcessing; CanMainFunctionWritePeriod. |
| Supporting artifact | Interrupt control and write-polling flow, including nested disable/enable behavior, interrupt-state restoration, polling-period behavior, and transmit confirmation forwarding |

## 7.1 Scope Summary

Assignment CAN-A05 covers CAN Driver interrupt control and transmit-confirmation polling. The relevant APIs are Can_DisableControllerInterrupts, Can_EnableControllerInterrupts, and Can_MainFunction_Write.

The interrupt-control scope includes controller-specific interrupt disabling, balanced nested disable and enable calls, restoration of interrupts permitted by the current software status, tracking of interrupt-state changes made by other CAN Driver functions, API reentrancy, and applicable development-error handling.

The write-polling scope includes configuration-dependent polling of TX confirmations, POLLING and MIXED processing, the permitted empty implementation when polling is unused, multiple polling periods, and notification of successful transmission.

The CAN Driver is responsible for controller-register access and for maintaining the information needed to restore the correct interrupt-enable state. The implementation of CanIf callbacks, CanSM, CanTp, CAN Transceiver behavior, and upper-layer transmission management are excluded.

Configuration dependencies include CanDevErrorDetect, CanTxProcessing, CanHardwareObjectUsesPolling, CanMainFunctionRWPeriodRef, CanMainFunctionRWPeriods, and CanMainFunctionPeriod.

## 7.2 Applicable SWS Requirements

| SWS Requirement ID | Short Description | Applicability | Note |
| --- | --- | --- | --- |
| SWS_Can_00231 | Definition of Can_DisableControllerInterrupts. | Applicable | Synchronous, reentrant API; Controller input; no return value. |
| SWS_Can_00049 | Disable all interrupts of the addressed controller when enabled. | Applicable | Controller-specific register behavior. |
| SWS_Can_00202 | Require balanced enable calls after repeated disable calls. | Applicable | Defines nested interrupt-control behavior. |
| SWS_Can_00204 | Track individual interrupt-state changes for correct restoration. | Applicable | Implementation mechanism is not prescribed. |
| SWS_Can_00205 | Report CAN_E_UNINIT for disable before initialization. | TBD | Depends on CanDevErrorDetect. |
| SWS_Can_00206 | Report CAN_E_PARAM_CONTROLLER for invalid controller on disable. | TBD | Depends on CanDevErrorDetect. |
| SWS_Can_00232 | Definition of Can_EnableControllerInterrupts. | Applicable | Synchronous, reentrant API; Controller input; no return value. |
| SWS_Can_00050 | Enable interrupts required by current software status. | Applicable | Subject to balanced-call behavior. |
| SWS_Can_00208 | No action for enable without a preceding disable. | Applicable | Protects the current interrupt state. |
| SWS_Can_00209 | Report CAN_E_UNINIT for enable before initialization. | TBD | Depends on CanDevErrorDetect. |
| SWS_Can_00210 | Report CAN_E_PARAM_CONTROLLER for invalid controller on enable. | TBD | Depends on CanDevErrorDetect. |
| SWS_Can_00225 | Definition of Can_MainFunction_Write. | TBD | Used when TX-confirmation polling is configured. |
| SWS_Can_00031 | Poll TX confirmations in POLLING or MIXED mode. | TBD | Depends on CanTxProcessing and CanHardwareObjectUsesPolling. |
| SWS_Can_00178 | Permit empty implementation when no polling is used. | TBD | Applies when TX polling is unused. |
| SWS_Can_00441 | Provide period-specific functions for multiple polling periods. | TBD | Depends on CanMainFunctionRWPeriods. |
| SWS_Can_00016 | Call CanIf_TxConfirmation after successful transmission. | TBD | Polling path applies when TX completion is detected by Can_MainFunction_Write. |

## 7.3 Project SRS Requirements

### 7.3.1 CAN_SRS_049  - Disable Enabled Controller Interrupts

| SRS ID | CAN_SRS_049 |
| --- | --- |
| Assignment ID | CAN-A05 |
| Title | Disable Enabled Controller Interrupts |
| Requirement statement | The CAN Driver shall access the addressed CAN controller registers to disable all interrupts of that controller only when interrupts for that controller are enabled. |
| Rationale | Prevent interrupt-driven processing for the addressed controller while avoiding unnecessary register modification when the interrupts are already disabled. |
| Precondition | The CAN Driver is initialized, Controller is valid, and interrupts for the addressed controller are enabled. |
| Trigger | Can_DisableControllerInterrupts(Controller) is called. |
| Input | Controller. |
| Expected behavior | All interrupts of the addressed controller are disabled; interrupts of other controllers are unchanged. |
| Error handling | DET: Report CAN_E_UNINIT when uninitialized and CAN_E_PARAM_CONTROLLER when Controller is out of range, if development-error detection is enabled. Runtime error: None specified. Return behavior: None; void API. |
| Verification method | Unit Test + Integration Test |
| SWS source ID | SWS_Can_00049; SWS_Can_00205; SWS_Can_00206 |
| Applicability | Applicable |
| Open Point / Assumption | See OP-A05-001 |

### 7.3.2 CAN_SRS_050  - Balance Nested Interrupt-Control Calls

| SRS ID | CAN_SRS_050 |
| --- | --- |
| Assignment ID | CAN-A05 |
| Title | Balance Nested Interrupt-Control Calls |
| Requirement statement | The CAN Driver shall keep the addressed controller interrupts disabled until Can_EnableControllerInterrupts has been called as many times as Can_DisableControllerInterrupts was previously called for that controller. |
| Rationale | Preserve the interrupt-disabled condition across repeated or nested requests. |
| Precondition | One or more unmatched disable calls exist for the addressed controller. |
| Trigger | Can_EnableControllerInterrupts(Controller) is called. |
| Input | Controller and the internally tracked disable/enable balance. |
| Expected behavior | Interrupts remain disabled until the enable and disable calls are balanced. |
| Error handling | DET: Enable API errors remain applicable. Runtime error: None specified. Return behavior: None; void API. |
| Verification method | Unit Test |
| SWS source ID | SWS_Can_00202 |
| Applicability | Applicable |
| Open Point / Assumption | See OP-A05-002 |

### 7.3.3 CAN_SRS_051  - Track Controller Interrupt State Changes

| SRS ID | CAN_SRS_051 |
| --- | --- |
| Assignment ID | CAN-A05 |
| Title | Track Controller Interrupt State Changes |
| Requirement statement | The CAN Driver shall track individual interrupt enabling and disabling performed by its other functions so that the correct interrupt-enable state can be restored. |
| Rationale | Prevent temporary controller-wide disabling from losing interrupt-state changes requested by other CAN Driver functions. |
| Precondition | Controller interrupts have been disabled through Can_DisableControllerInterrupts. |
| Trigger | Another CAN Driver function changes the intended state of an individual interrupt. |
| Input | Individual interrupt-state changes made by CAN Driver functions. |
| Expected behavior | The tracked state represents the interrupt set to restore when re-enabling becomes eligible. |
| Error handling | DET: None specified. Runtime error: None specified. Return behavior: Not applicable. |
| Verification method | Analysis + Unit Test |
| SWS source ID | SWS_Can_00204 |
| Applicability | Applicable |
| Open Point / Assumption | See OP-A05-002 |

### 7.3.4 CAN_SRS_052  - Restore Allowed Controller Interrupts

| SRS ID | CAN_SRS_052 |
| --- | --- |
| Assignment ID | CAN-A05 |
| Title | Restore Allowed Controller Interrupts |
| Requirement statement | The CAN Driver shall enable all interrupts required by the current software status when the interrupt-control calls for the addressed controller are balanced. |
| Rationale | Restore the currently valid interrupt configuration instead of an obsolete register state. |
| Precondition | The CAN Driver is initialized, Controller is valid, and the current enable call balances preceding disable calls. |
| Trigger | Can_EnableControllerInterrupts(Controller) is called. |
| Input | Controller and current software interrupt status. |
| Expected behavior | All interrupts required by current software status are enabled for the addressed controller. |
| Error handling | DET: Report CAN_E_UNINIT when uninitialized and CAN_E_PARAM_CONTROLLER when Controller is out of range, if development-error detection is enabled. Runtime error: None specified. Return behavior: None; void API. |
| Verification method | Unit Test + Integration Test |
| SWS source ID | SWS_Can_00050; SWS_Can_00209; SWS_Can_00210 |
| Applicability | Applicable |
| Open Point / Assumption | See OP-A05-001 |

### 7.3.5 CAN_SRS_053  - Ignore Enable Without Prior Disable

| SRS ID | CAN_SRS_053 |
| --- | --- |
| Assignment ID | CAN-A05 |
| Title | Ignore Enable Without Prior Disable |
| Requirement statement | The CAN Driver shall perform no interrupt-state action when Can_EnableControllerInterrupts is called for a controller for which Can_DisableControllerInterrupts has not previously been called. |
| Rationale | Prevent an unmatched enable call from altering the current interrupt state. |
| Precondition | The CAN Driver is initialized, Controller is valid, and no unmatched disable call exists. |
| Trigger | Can_EnableControllerInterrupts(Controller) is called. |
| Input | Controller and tracked interrupt-control call state. |
| Expected behavior | The addressed controller interrupt-enable state remains unchanged. |
| Error handling | DET: Normal enable API initialization and parameter checks remain applicable; no error is specified for the unmatched enable condition. Runtime error: None specified. Return behavior: None. |
| Verification method | Unit Test |
| SWS source ID | SWS_Can_00208 |
| Applicability | Applicable |
| Open Point / Assumption | None. |

### 7.3.6 CAN_SRS_054  - Provide Reentrant Interrupt Control

| SRS ID | CAN_SRS_054 |
| --- | --- |
| Assignment ID | CAN-A05 |
| Title | Provide Reentrant Interrupt Control |
| Requirement statement | The CAN Driver shall implement Can_DisableControllerInterrupts and Can_EnableControllerInterrupts as reentrant services. |
| Rationale | Support calls from reentrant functions without corrupting shared interrupt-control state. |
| Precondition | The interrupt-control APIs are included in the CAN Driver implementation. |
| Trigger | Overlapping or nested interrupt-control API executions occur. |
| Input | Controller supplied to each invocation. |
| Expected behavior | Each invocation maintains correct controller-specific behavior without corrupting shared state. |
| Error handling | DET: API-specific initialization and controller errors apply. Runtime error: None specified. Return behavior: Both APIs return void. |
| Verification method | Analysis + Unit Test |
| SWS source ID | SWS_Can_00231; SWS_Can_00232 |
| Applicability | Applicable |
| Open Point / Assumption | See OP-A05-002 |

## 7.4 Supporting Artifact

CAN-A05 Interrupt Control Table

| Case | Precondition | Trigger | Expected Result | SRS ID | SWS ID |
| --- | --- | --- | --- | --- | --- |
| First disable | Initialized; valid controller; interrupts enabled | Can_DisableControllerInterrupts | Disable all interrupts of the addressed controller. | CAN_SRS_049 | SWS_Can_00049 |
| Repeated disable | At least one unmatched disable call | Can_DisableControllerInterrupts | Keep interrupts disabled and update tracked balance. | CAN_SRS_050 | SWS_Can_00202 |
| Partial enable | More than one unmatched disable call | Can_EnableControllerInterrupts | Keep interrupts disabled. | CAN_SRS_050 | SWS_Can_00202 |
| Balanced enable | One unmatched disable call | Can_EnableControllerInterrupts | Restore interrupts required by current software status. | CAN_SRS_052 | SWS_Can_00050; SWS_Can_00202 |
| Unmatched enable | No unmatched disable call | Can_EnableControllerInterrupts | Do not alter interrupt state. | CAN_SRS_053 | SWS_Can_00208 |
| Disable before initialization | Driver uninitialized; DET enabled | Can_DisableControllerInterrupts | Report CAN_E_UNINIT. | CAN_SRS_049 | SWS_Can_00205 |
| Invalid controller on enable | Controller out of range; DET enabled | Can_EnableControllerInterrupts | Report CAN_E_PARAM_CONTROLLER. | CAN_SRS_052 | SWS_Can_00210 |

Open points for the artifact: concrete interrupt sources and registers, balancing mechanism, initial interrupt states, register observation method, and project configuration values are not supplied.

## 7.5 Open Points and Assumptions

| ID | Description | Impact | Status |
| --- | --- | --- | --- |
| OP-A05-001 | Project-specific interrupt sources and register mapping are not supplied. | Hardware-level verification cannot be finalized. | Open |
| OP-A05-002 | The source permits but does not mandate a counter for nested control. | Internal design cannot be prescribed. | Open |
| OP-A05-003 | CanDevErrorDetect project value is not supplied. | DET-related applicability remains TBD. | Open |
| OP-A05-004 | CanTxProcessing project value is not supplied. | Write polling applicability remains TBD. | Open |
| OP-A05-005 | CanHardwareObjectUsesPolling values are not supplied. | Objects polled in MIXED mode cannot be listed. | Open |
| OP-A05-006 | Polling-period number and ShortName values are not supplied. | Period-specific function names cannot be finalized. | Open |
| OP-A05-007 | Project TX-completion observation mechanism is not supplied. | Integration-test details cannot be finalized. | Open |
| OP-A05-008 | Project policy for AUTOSAR may-statements is not supplied. | CAN_A05_SRS_008 remains TBD. | Open |